| MIFAB, Inc. | July 2008 |
| --- | --- |
| 1321 West 119th Street |  |
| Chicago, IL 60643-5109 |  |
| Toll Free (800) 465-2736 |  |
| Phone (773) 341-3030 | E-Mail mail@mifab.com |
| Fax (773) 341-3049 | Web Site www.mifab.com |

Product Guide Specification

Specifier Notes: This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format. The section must be carefully reviewed and edited by the Architect to meet the requirements of the project and local building code. Coordinate this section with other specification sections and the drawings. Delete all “Specifier Notes” when editing this section.

SECTION 22160

ROOF DRAINS

Specifier Notes: This section covers MIFAB “R1200” large sump roof drain. Consult MIFAB for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Large sump roof drains.

1.2	RELATED SECTIONS

Specifier Notes: Edit the following list of related sections as required for the project. List other sections with work directly related to this section.

A.	Section 22100 - Building Services Piping.

1.3	REFERENCES

A.	ANSI/ASME A112.21.2M - Roof Drains.

B.	ASTM A 48 - Standard Specification for Gray Iron Castings.

1.4	SUBMITTALS

A.	Comply with Section 01330 - Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, including installation instructions.

1.5	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer's instructions.

C.	Handling:  Protect materials from damage during handling and installation.

PART 2	PRODUCTS

2.1	MANUFACTURER

Specifier Notes: Specify MIFAB, Inc. in the US or MIFAB Canada in Canada

A.	[MIFAB, Inc., 1321 West 119th Street, Chicago, IL. 60643-5109  Toll Free (800) 465-2736.]

Web Site www.mifab.com  E-Mail mail@mifab.com.

2.2	MATERIALS

A.	Roof Drains:  R1200 large sump roof drain.

1.	Compliance:  ANSI/ASME A112.21.2M.

2.	Body:  A2 deep sump body.  Lacquered, ASTM A 48, Class 25 cast iron body with anchor flange.

3.	Dome Strainer:  Self-locking polydome strainer.  Free area of 136 square inches (87,741 mm2).

4.	Membrane Clamp Ring:  2-5/16-inch (60-mm) wide, ASTM A 48, Class 25 cast iron, waterproofing membrane clamp ring with integral gravel stop.

Specifier Notes: Specify pipe size.

5.	Pipe Size:  [2 inches (50 mm)]  [3 inches (75 mm)]  [4 inches (100 mm)]  [6 inches (150 mm)]  [8 inches (200 mm)].

Specifier Notes: Specify required roof drain options and revise above text as necessary. Delete options not required. Consult MIFAB for additional information.

B.	Roof Drain Options:

1.	3M epoxy finish.

2.	Vandal-proof dome.

3.	Acid-resistant epoxy coating.

4.	Dome:  [Galvanized metal]  [Rough bronze]  [Ductile iron]  [Low silhouette aluminum].

5.	All galvanized.

6.	Body:  [Bronze]  [Type 304 stainless steel]  [Type 316 stainless steel].

7.	Socket Connection Body:  [PVC]  [ABS].

8.	Mesh Screen Over Dome:  [Bronze] [Stainless steel].

9.	Threaded side outlet.

10.	Sump receiver.

11.	Secondary clamp.

12.	Adjustable extension.

13.	Accuflow, parabolic weir.

14.	Ballast guard.

15.	Ballast guard with secondary clamp.

16.	Outlet:  [Push-on]  [Threaded]  [Inside caulk].

17.	2-inch external water dam.

18.	Underdeck clamp.

19.	Fixed static extension, 2 inches, 3 inches, 4 inches.

20.	Adjustable water dam.

21.	Waterproofing flange.

22.	Extended wide deck flange.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive roof drains.  Notify Architect of conditions that would adversely affect installation or subsequent use.  Do not proceed with installation until unsatisfactory conditions are corrected.

3.2	INSTALLATION

A.	Install roof drains in accordance with manufacturer's instructions at locations indicated on the drawings.

B.	Install roof drains plumb, level, and to correct elevation.

C.	Install roof drains using manufacturer's supplied hardware.

3.3	PROTECTION

A.	Protect installed roof drains from damage during construction.

END OF SECTION